WriteAtr V0.91 beta (c) 2002 by HiassofT

This program allows you to transfer double density (180k) ATR-images

directly from/to Atari-XL floppy disks with your PC!

All you need is a PC running MSDOS and a 5.25" floppy drive in your PC!

******************** ATTENTION ******* BETA SOFTWARE ********************

Please note that WriteAtr is still beta. This means I cannot guarantee

that it will work properly, I don't even know if it will work at all

on your computer! All I can say is that it worked for me so far

(except for the support for higher capacity formats, which I have not

been able to test at all yet).

I would be glad if you could send me some feedback if the program worked

on your PC or if you encountered problems. In the case of problems,

please use the debugging switch (see below) an send me the output

of WriteAtr. The easiest way to accomplish this if you use the

output-redirection built into DOS. Simply type:

writeatr -d myfile.atr > debug.txt

and email me the file debug.txt. (of course, substitute myfile.atr

with the atr-file you were using, and don't forget to add the other

options you were using).

******************** ATTENTION ******* BETA SOFTWARE ********************

You are allowed to use, copy, and distribute this program for free.

If you like this program, or if you have suggestions for new features,

feel free to contact me:

HiassofT

Matthias Reichl

Wolfgangerstr. 26

A-4820 Bad Ischl

Austria

e-mail: hias@horus.com

WWW: http://www.horus.com/~hias/atari/

1. Installation:

There should be 2 files in the zip-file:

writeatr.exe - the executable

writeatr.doc - this file

change.log   - ChangeLog of WriteAtr

To install the program, simply copy the file writeatr.exe

into any directory (eg c:\tools).

2. Using the program:

WriteAtr does not have a graphical user interface. Instead of that,

you have to pass commandline options to WriteAtr to tell it what

it should do. Don't be afraid - there are only very few options

and you will know them all in 2 minutes.

If you start WriteAtr without any options, it will print out an

overview of the available options. It should look like this:

Usage: writeatr [-aX] [-b] [-d] [-f] [-l] [-r] filename.atr

options:

-b : use drive B

-d : turn on debugging

-f : fast mode - don't format disk when writing

-l : create long ATR-image (extra 384 zero-bytes at the end)

-r : read disk (create ATR-image of disk)

-aX : UNTESTED: set atari floppy drive type (X=0..4)

0=180k/5.25" 1=360k/5.25" 2=360k/5.25"/XF551

3=720k/3.5" 4=1.44M/3.5"

-aX : UNTESTED: set atari floppy drive type (X=0..4, see manual)

The only mandatory item in the commandline is the filename of the

ATR file. You can combine several options, although some combinations

will have no effect (eg: fast mode makes no sense when reading a

disk).

3. Writing ATR-images to floppy disk:

By default, if you don't specify any other options, WriteAtr

will read the given ATR-File and write it to the disk in

your first floppy drive (drive A). The disk does not need to be

formatted, it will be formatted on the fly.

For example:

writeatr d:\hias\xl\dos\mydos.atr

If your 5.25" floppy drive is installed as drive B, you have to add

the option "-b".

For example:

writeatr -b d:\hias\xl\source\disk42.atr

If your disk has already been formatted by WriteAtr, you can save

a little time by disabling the on-the-fly formatting with the

"-f" option. But beware, if your floppy disk has not been formatted

in your PC-drive, write errors might occur!

For example:

writeatr -b -f c:\temp\backup.atr

4. Creating an ATR-image from a floppy disk:

Creating an image is as easy as writing images: Simply use the

option "-r". WriteAtr will then read the floppy disk and store

an ATR-image in the given file. NOTE: If the file already

exists, it will be overwritten!

For example:

writeatr -r pics.atr

or:

writeatr -b -r basic.atr

If you use the option "-l" when creating an ATR-image, WriteAtr

will create a (NumberOfSectors*256+16) bytes long ATR-file. By

default, WriteAtr creates a 384 bytes smaller ATR-files. The only

difference between these two types of images is that the larger

image is padded with 384 zero-bytes at the end of the file.

The main purpose of this option was for debugging. If you dump

an ATR-image to disk, and then read it back, it should (of course)

be exactly the same as the original image (except maybe some bits

in the header). With the "-l" option you can create an image that

has the same size as the original image, so that a file-compare

(eg.: "fc /b orig.atr new.atr") will work.

For example:

writeatr -b -l -r new.atr

will create a long ATR-image from drive B.

5. Higher capacity formats **** ATTENTION **** UNTESTED ****

I have built preliminary support for formats higher than 180k

into version 0.91beta of WriteAtr. Please note, that this code

is COMPLETELY UNTESTED, since I don't have access to a XF551

or a 3.5" disk drive for my Atari at the moment.

******************  If you test this code,  *****************

******************  please, please, please  *****************

***************  send me feedback if it works!  *************

*  Especially send me the exact name and manufacturer of    *

*  your atari disk drive, the kind of floppy drive in your  *

*  PC (eg. 360/720/1.2/1.44MB 3.5/5.25") and the kind of    *

*  problem you were experiencing.                           *

*************************  THANKS!  *************************

You can select the type of your Atari floppy drive by using the

"-aX" command line option. Substitute 'X' with a value from

0 to 4. The different values have the following meaning:

-a0 : single sided, double density, 5.25", 180k drive (eg a 1050).

256 bytes per sector, 18 sectors per track, 40 tracks per side.

This is also the default that is used if you don't specify

the "-aX" option (so this option is pretty useless :-)

-a1 : double sided, double density, 5.25", 360k drive (NOT a XF551!)

256 bytes per sector, 18 sectors per track, 40 tracks per side.

Sectors 1-720 are stored on side 1 in tracks 0-39.

Sectors 721-1440 are stored on side 2 in tracks 0-39.

-a2 : double sided, double density, 5.25", 360k Atari XF551 drive

256 bytes per sector, 18 sectors per track, 40 tracks per side.

Sectors 1-720 are stored on side 1 in tracks 0-39.

Sectors 721-1440 are stored on side 2 in tracks 39-0

(note the reversed track-order on side 2!)

-a3 : double sided, double density, 3.5", 720k drive

256 bytes per sector, 18 sectors per track, 80 tracks per side.

Sectors 1-1440 are stored on side 1 in tracks 0-79.

Sectors 1441-2880 are stored on side 2 in tracks 0-79.

-a4 : double sided, high density, 3.5", 1.44M drive

256 bytes per sector, 36 sectors per track, 80 tracks per side.

Sectors 1-2880 are stored on side 1 in tracks 0-79.

Sectors 2881-5760 are stored on side 2 in tracks 0-79.

6. Debugging:

If you include the option "-d", WriteAtr will print out some info

about the given ATR-image when writing images.

If you have problems using WriteAtr, please use this option and

email the output of WriteAtr to me. Here's my email address:

hias@horus.com

7. Features / known bugs:

WriteAtr currently only supports double density (256 bytes per

sector, 720 sectors per disk) ATR-images. Support for higher

capacity formats exists, but is untested.

If the ATR-file contains fewer sectors than your floppy disk,

WriteAtr fills remaining sectors with zeroes.

WriteAtr only works in plain MSDOS, because it needs direct access

to the PC-hardware. Using WriteAtr inside a Windows-DOS-box

might work, but I can't guarantee it.

It is not possible to use this program without a harddisk, because

it has to reprogram the disk controller to read atari disks and

the controller is no longer able to read PC-disks (at the end

of the program the controller is reset so that you can read PC-

disks again).

Errors can occur when you use disks formatted with the Atari XL

on a PC-system with a highdensity disk drive. This is because the

Atari spins disks with 288 rpm and HD-drives spin them with 360 rpm.

It is better to let WriteAtr format the disks in your PC drive.

It is not possible to access side 2 of XL-disks by flipping them,

because all PC disk drives use the index-hole on the disk

(the XL doesn't). The only way would be to cut a second index-hole

in your disk (a similar problem occurred with the XF 551-drive

because this drive also uses the index-hole).