WriteAtr V0.92 (c) 2002 by HiassofT

This program allows you to transfer double density (256 bytes per

sector) ATR-images directly from/to Atari-8bit floppy disks with your PC!

All you need is a PC running MSDOS and a 5.25" or 3.5" floppy

disk drive in your PC!

You are allowed to use, copy, and distribute this program for free.

If you like this program, or if you have suggestions for new features,

feel free to contact me:

HiassofT

Matthias Reichl

Wolfgangerstr. 26

A-4820 Bad Ischl

Austria

e-mail: hias@horus.com

WWW: http://www.horus.com/~hias/atari/

1. Installation:

There should be 3 files in the zip-file:

writeatr.exe - the executable

writeatr.doc - this file

change.log   - ChangeLog of WriteAtr

To install the program, simply copy the file writeatr.exe

into any directory (eg c:\tools).

2. Using the program:

WriteAtr does not have a graphical user interface. Instead of that,

you have to pass commandline options to WriteAtr to tell it what

it should do. Don't be afraid - there are only very few options

and you will know them all in 2 minutes.

If you start WriteAtr without any options, it will print out an

overview of the available options. It should look like this:

Usage: writeatr [-b] [-d] [-fX] [-l] [-n] [-r] filename.atr

options:

-b : use drive B

-fX : set atari floppy drive type (X=0..8)

0=180k/5.25" (this is the default)

1=360k/5.25"        2=720k/3.5"        3=1.44M/3.5"

4=360k/5.25"/head   5=720k/3.5"/head   6=1.44M/3.5"/head

7=360k/5.25"/XF551  8=720k/3.5"/XF551

-r : read disk (create ATR-image of disk)

-l : create long ATR-image (extra 384 zero-bytes at the end)

-d : increase debug level by 1

-n : no verify when writing (DANGEROUS!)

The only mandatory item in the commandline is the filename of the

ATR file. You can combine several options, although some combinations

will have no effect (eg: "no verify writes" makes no sense when reading

a disk).

By default, WriteAtr will assume your drive is a 5.25" disk drive

and the floppy disk should be formatted in the standard 180k

double density format (720 sectors of 256 bytes each).

If you want to use higher capacity formats (up to 1.44MB) please

have a look into section 6.

3. Writing ATR-images to floppy disk:

By default, if you don't specify any other options, WriteAtr

will read the given ATR-File and write it to the disk in

your first floppy drive (drive A). The disk does not need to be

formatted, it will be formatted on the fly.

For example:

writeatr d:\hias\xl\dos\mydos.atr

If you want to use drive "B", you have to add the option "-b".

For example:

writeatr -b d:\hias\xl\source\disk42.atr

By default WriteAtr verifies the sectors after writing. Of course

this takes a little time, but you can be sure that everything got

written correctly.

If you like doing risky things and want to save a little bit

time, you might consider disabling the write-verify by using

the "-n" option. I don't recommend this, so do it at your

own risk!

For example:

writeatr -b -n d:\hias\xl\games\coolgame.atr

4. Creating an ATR-image from a floppy disk:

Creating an image is as easy as writing images: Simply use the

option "-r". WriteAtr will then read the floppy disk and store

an ATR-image in the given file. NOTE: If the file already

exists, it will be overwritten!

For example:

writeatr -r pics.atr

or:

writeatr -b -r basic.atr

If you use the option "-l" when creating an ATR-image, WriteAtr

will create a (NumberOfSectors*256+16) bytes long ATR-file. By

default, WriteAtr creates a 384 bytes smaller ATR-files. The only

difference between these two types of images is that the larger

image is padded with 384 zero-bytes at the end of the file.

The main purpose of this option was for debugging. If you dump

an ATR-image to disk, and then read it back, it should (of course)

be exactly the same as the original image (except maybe some bits

in the header). With the "-l" option you can create an image that

has the same size as the original image, so that a file-compare

(eg.: "fc /b orig.atr new.atr") will work. Such (longer) images

are created by SIO2PC (at least in the versions I used), but in

99 percent of the cases you won't need this option at all.

For example:

writeatr -b -l -r new.atr

will create a long ATR-image from drive B.

5. Error handling

Whenever an operation (formatting a track, reading/writing/verifying

a sector) fails, the exact position and the exact BIOS error code

are reported and you have the possibility to abort, retry, or ignore

the failed operation. Whenever a sector-IO operation fails, WriteAtr

reports both the (PC-style) sector/track/head position, and the

position translated into an (Atari-style) sector number.

If you choose "abort" the program will stop immediately,

"retry" will try the same operation again, and "ignore" will

skip the current operation and proceed to the next.

Please note: Ignoring errors is generally a very bad idea!

In most cases errors are caused by bad floppy disks. So if you

experience an error while writing an image to a disk, please try

using another disk.

On the other hand, if you are getting read errors while creating

an image from a disk, there is a high possibility that your floppy

disk is starting to "fade away". In this case, you should retry the

failed operation until it succeeds. If you just ignore the error,

you will be creating a broken image! So use this at your own risk!

6. Higher capacity formats

You can select the type of your Atari floppy drive by using the

"-fX" command line option. Substitute 'X' with a value from

0 to 8. The different values have the following meaning:

-f0 : single sided, double density, 5.25", 180k drive (eg a 1050).

256 bytes per sector, 18 sectors per track, 40 tracks per side.

This is also the default that is used if you don't specify

the "-fX" option (so this option is pretty useless :-)

-f1 : double sided, double density, 5.25", 360k drive (default mapping)

256 bytes per sector, 18 sectors per track, 40 tracks per side.

-f2 : double sided, double density, 3.5", 720k drive (default mapping)

256 bytes per sector, 18 sectors per track, 80 tracks per side.

-f3 : double sided, high density, 3.5", 1.44M drive (default mapping)

256 bytes per sector, 36 sectors per track, 80 tracks per side.

-f4 : double sided, double density, 5.25", 360k drive, head first

256 bytes per sector, 18 sectors per track, 40 tracks per side.

-f5 : double sided, double density, 3.5", 720k drive, head first

256 bytes per sector, 18 sectors per track, 80 tracks per side.

-f6 : double sided, high density, 3.5", 1.44M drive, head first

256 bytes per sector, 36 sectors per track, 80 tracks per side.

-f7 : double sided, double density, 5.25", 360k Atari XF551 drive

256 bytes per sector, 18 sectors per track, 40 tracks per side.

-f8 : double sided, double density, 3.5", 720k drive built into XF551

256 bytes per sector, 18 sectors per track, 80 tracks per side.

Short Explanation of the various mapping-types:

* Most double sided Atari disk drives (like the HDI or the ATR8000)

first fill tracks 0-39 (or 0-79) on the first side, then switch

over to side 2 and again fill tracks 0-39 (0-79 for 3.5" disks).

If you have got such a drive, use one of the options

"-f1", "-f2", or "-f3".

* If -f1/-f2/-f3 didn't work for you, please try one of the

options "-f4", "-f5", or "-f6". This mapping is not too common

for Atari disk drives, the only drive I know of that uses this

mapping is the "Karin Maxi".

If your Atari disk drive uses this mapping, it first fills

track 0 on side 1, then track 0 on side 2, then seeks to track 1,

again first fills track 1 / side 1, then track 1 / side 2,

and so on, until it finishes with track 39 (79) / side 2.

I called this mapping 'head first' (or short: head), because

the drives switches the heads (sides) first before switching

the track.

* If you have got a XF551, use options "-f7" or "-f8".

The XF551 first fills track 0 on the first side. Then it

fills track 1, then track 2, ... up to track 39 (on a 5.25" disk)

or 79 (on a 3.5" disk). Then it switches to side 2 and fills

the disk in reverse order (starting at track 39/79, then 38/78,

... til it has reached the end of the disk at track 0).

If you've got a drive other than the Atari XF551, please tell me

which mapping (default or "head first") worked for you. Email me the

manufacturer and the name of your drive, so I can build a list of

the known drives and mappings and include it in future versions

of WriteAtr.

Thanks!

Long explanation of the different mapping types:

(you may skip it if you are already familiar with the concept

of SIO and disk drives).

The OS of the Atari accesses the sectors of a disk in a linear,

block oriented, way. It does not care if you've got a single

or double sided floppy drive, if it's a 3.5" or a 5.25" drive

or if you connected your Atari to something that's only pretending

to be a floppy drive (eg your PC running SIO2PC or APE).

In other words: if the Atari wants to write to a disk it simply

tells the drive "hey, drive, here's some data. please be so

kind and write it to sector 528". When, later, the Atari tells

the drive to read back sector 528, the drive should return the

same data that the Atari sent to the drive when writing

(otherwise we have kind of serious troubles :-)

On the other hand, the drive has of course to know much more

about the floppy disk that it's going to access. Usually a

floppy disk is accessed by specifying the HEAD (eg: a double sided

floppy disk drive has two heads, head "0" acessing the first side,

and head "1" accessing the second side), the TRACK (usually disks

have 40 or 80 tracks), and the SECTOR-number within this track,

on the specified side (head).

And that's the point where the troubles begin.

It's the job of the disk drive to translate each sector-number

(as received from the Atari) into a unique sector/track/head-triple.

Let's start with the simplest case - a single sided/double density 1050:

A 1050 can store 720 sectors of 256 bytes each on a disk. Actually,

such a disk consists of 40 tracks of 18 sectors:

Sectors 1-18 are stored in sectors 1-18 of track 0.

Sectors 19-36 are stored in sectors 1-18 of track 1.

...

Sectors 703-720 are stored in sectors 1-18 of track 39.

Simple, wasn't it?

Now let's move to the complicated part - double sided disks.

I would like to show you the difference between the various

mappings on a simple example: a double sided, double density,

360k 5.25" disk, formatted with 40 tracks of 18 sectors each.

This means we have a total of 1440 sectors of 256 bytes.

* The default mapping:

This mapping stores the first 720 sectors on the first side in

tracks 0-39, like the 1050 or the XF551.

When finished with the first side, the drive switches to the second

side and starts again at sector 1 of track 0. This means:

Sector 721 is stored in sector 1 of track 0 on side 1.

Sector 722 is stored in sector 2 of track 0 on side 1.

...

Sector 738 is stored in sector 18 of track 0 on side 1.

Sector 739 is stored in sector 1 of track 1 on side 1.

...

Sector 1440 is stored in sector 18 of track 39 on side 1.

* The head-first mapping:

This mapping is identically to the way the PC stores data on

a floppy disk. First the drive fills track 0 on side 0. Then

it switches to side 1 and fills track 0. When it's finished

with that, it seeks forward to track 1. There it again first

fills the track on side 0, then on side 1.

Sector 1 is stored in sector 1 of track 0 on side 0.

Sector 2 is stored in sector 2 of track 0 on side 0.

...

Sector 18 is stored in sector 18 of track 0 on side 0.

Sector 19 is stored in sector 1 of track 0 on side 1.

...

Sector 36 is stored in sector 18 of track 0 on side 1.

....

Sector 1405 is stored in sector 1 of track 39 on side 0.

...

Sector 1422 is stored in sector 18 of track 39 on side 0.

Sector 1423 is stored in sector 1 of track 39 on side 1.

...

Sector 1440 is stored in sector 18 of track 39 on side 1.

* The XF551 mapping:

This mapping is really strange :-)

I just can't imagine that someone thought this was a good idea.

And even more unrealistic than that, someone actually used it

in a floppy disk drive - the XF551.

Older (single sided) drives, like the 1050, should be able to access

at least the first half of the disk. Therefore sectors 1-720 are

stored on the first side, in the same way as the 1050 would do it:

Sectors 1-18 in sectors 1-18 of track 0 on side 0.

...

Sectors 703-720 in sectors 1-18 of track 39 on side 0.

When finished with the first side, the XF551 switches over to side 2.

But it does not start again at track 0, no, it starts at the last

track. And that's not enough, even the sector numbers within each

track are reversed! This means:

Sector 721 is stored in sector 18 of track 39 on side 1.

Sector 722 is stored in sector 17 of track 39 on side 1.

...

Sector 738 is stored in sector 1 of track 39 on side 1.

Sector 739 is stored in sector 18 of track 38 on side 1.

...

Sector 1439 is stored in sector 2 of track 0 on side 1.

Sector 1440 is stored in sector 1 of track 0 on side 1.

7. Debugging:

If you include the option "-d", WriteAtr will print out some

debug info when reading/writing images.

If you specify the option "-d" twice (eg: "writeatr -d -d test.atr")

you will get even more debugging info.

If you have problems using WriteAtr, please use this option and

email the output of WriteAtr to me. Since the output will usually

be quite long, use output redirection to save the output into a file.

For example:

writeatr -d -d test.atr > debug.log

Here are some hints how you can test if WriteAtr works.

* Format a disk on your Atari using MyDOS and copy some large

files onto the disk, so that at least more than half of the

sectors of the disk are used.

Then read this disk with WriteAtr on your PC and either

- use SIO2PC with the ATR-image or

- load the ATR-image into an emulator

and check if you can access all of the files.

If you chose the wrong mapping, or if there's a bug in WriteAtr

MyDos will give you an error message concerning incorrect sector

chaining. If none of the mappings offered by WriteAtr work for

you, please contact me by email and, if possible, send me the

ATR-image and some information about the disk drive connected

to your Atari.

* If you've got some known good ATR-images (eg. images that work

with SIO2PC or an emulator), try to write them to an Atari

floppy using WriteAtr and then try reading them with your

Atari disk drive.

8. Features / known bugs:

WriteAtr only supports double density (256 bytes per sector)

ATR-images. This limitation is caused by the floppy disk controller

used in most PCs: the controller simply cannot cope with 128

bytes-per-sector formats.

If the size of the ATR-file does not match the size of the

selected disk type, WriteAtr will issue a warning but still do

what you told it to do. It is therefore possible to write

the first part of a larger ATR-file to a smaller disk, or

to write a smaller ATR-file to a larger disk. In the second

case, WriteAtr will fill the remaining sectors with zero bytes.

WriteAtr only works in plain MSDOS, because it needs direct access

to the PC-hardware. Using WriteAtr inside a Windows-DOS-box

might work (I had success using it in a Win95 and WinME DOS-box,

and also got several emails from people who had success), but

I can't guarantee that it will work on all PCs.

It is not possible to use this program without a harddisk, because

it has to reprogram the disk controller to read Atari disks and

the controller is no longer able to read PC-disks (at the end

of the program the controller is reset so that you can read PC-

disks again).

It is not possible to access side 2 of Atari-disks by flipping them,

because all PC disk drives use the index-hole on the disk

(eg the 1050 doesn't). The only way would be to cut a second index-hole

in your disk (a similar problem exists with the XF551-drive

because this drive also uses the index-hole).

If the specified disk format does not match the floppy disk inserted

into your PC disk drive (eg. if you insert a 3.5" 1.44M HD disk and

tell WriteAtr it is a 3.5" 720k DD disk) the floppy controller may

get confused, and further operations (even with the right format

specification) might fail. If this happens to you, simply take the

disk out of the drive and insert it again.

I'm not sure what's actually going wrong, but re-inserting the disk

always helped for me...

9. Credits

I would like to thank a lot of people who helped me develop WriteAtr

(by providing me important information, helped me testing, etc.).

Many, many thanks go to (in no particular order):

Lee Barnes

Erhard Puetz

Marco Antonio Checa Funcke

Mr.Bacardi

Ron Fucci

Larry White

HatMouse

Piotr Fusik

If I forgot _you_ and you should be on this list, just send me an email!

Thank you!

#